5-Stage CMOS Ring Oscillator Design and Analysis using LTspice (TSMC 180 nm)

1. Objective

The objective of this project was to design and simulate a 5-stage CMOS Ring Oscillator using TSMC 180 nm BSIM3 (Level-49) transistor models in LTspice. The design aims to generate a stable oscillation without requiring an external clock source while analysing its frequency, propagation delay, and startup behaviour.

2. Design Specifications

| Parameter | Value |
| --- | --- |
| Technology | TSMC 180 nm CMOS |
| Simulator | LTspice XVII |
| CMOS Model | BSIM3 Level-49 |
| Supply Voltage | 1.8 V |
| Number of Inverters | 5 |
| Oscillator Type | Odd-stage CMOS Ring Oscillator |
| PMOS Length | 180 nm |
| PMOS Width | 2 μm |
| NMOS Length | 180 nm |
| NMOS Width | 1 μm |
| Output Load | 1 fF |
| Simulation Time | 100 ns |

3. Theory

A ring oscillator consists of an odd number of CMOS inverters connected in a closed loop.

Each inverter contributes:

- One inversion (180° phase shift)

- One propagation delay

For oscillation,

- Total phase shift = 360°

- Loop gain ≥ 1

Since five inverters produce:

5×180∘=900∘

which is equivalent to

900∘=720∘+180∘

and the propagation delay contributes the remaining phase shift, sustained oscillation occurs.

The oscillation frequency is

f=12Ntp

where

N = Number of inverter stages

tp = propagation delay of one stage

4. Circuit Implementation

The oscillator consists of

- Five CMOS inverters

- PMOS connected to VDD

- NMOS connected to Ground

- Output of fifth inverter connected back to first inverter input

No external clock source is used.

Oscillation starts due to the inherent propagation delay of the inverter chain.

5. Challenges Encountered

Several issues were encountered during the design process.

Initial Problems

- Incorrect model library inclusion

- Unsupported BSIM4 model

- Wrong transistor model names

- LTspice file path issues

- Startup convergence problems

- Perfect symmetry causing delayed startup

- Incorrect transient analysis settings

6. Modifications Performed

The following improvements were implemented.

(a) Technology Model

Initially,

Level-1 MOS models

were used.

These were replaced with

TSMC180nm BSIM3 Level-49

models for industry-level accuracy.

(b) Correct Model Names

NMOS

TSMC180nmN

PMOS

TSMC180nmP

(c) Transistor Dimensions

Final dimensions selected:

NMOS

L = 180 nm

W = 1 μm

PMOS

L = 180 nm

W = 2 μm

Reason:

PMOS mobility is approximately half of NMOS mobility.

Doubling PMOS width balances pull-up and pull-down strengths.

(d) Supply Voltage

VDD = 1.8 V

This matches the nominal operating voltage of TSMC 180 nm CMOS technology.

(e) Simulation Command

Final transient simulation

.tran 0 100n

(f) Output Capacitor

A small

1 fF

capacitive load was connected to the output node to represent parasitic loading.

7. Final Waveform Analysis

The final simulation shows

- Stable oscillation

- Continuous oscillation

- Rail-to-rail output swing

- Proper phase shift between adjacent stages

- Uniform oscillation frequency

The startup delay is approximately

2 ns

which is expected because oscillation begins only after the signal propagates around the loop.

8. Measured Results

Oscillation Period

Measured using

.meas

Result

T=6.097×10-10  s

or

T=0.6097 ns

Oscillation Frequency

Measured

f=1T

Result

f=1.640 GHz

Propagation Delay

Measured

tp=57.1 ps

9. Theoretical Verification

Theory

f=12Ntp

Given

N=5

tp=57.1 ps

Therefore

f=12(5)(57.1×10-12)
f=1.75 GHz

Measured

1.64 GHz

Difference

≈6%

This difference is expected because the theoretical equation ignores

- Junction capacitances

- Wiring parasitics

- BSIM nonlinear effects

- Output loading

- Finite rise/fall times

Hence, the measured result closely agrees with theory.

10. Advantages

- Extremely simple architecture

- No external clock required

- Fully digital implementation

- Small silicon area

- High oscillation frequency

- Easily scalable

- Used in modern ASICs and SoCs

11. Applications

- PLLs

- Clock generation

- Process monitors

- Temperature sensors

- Voltage Controlled Oscillators

- Random Number Generators

- Built-In Self-Test (BIST)

- Time-to-Digital Converters

- PUFs (Physical Unclonable Functions)

12. Limitations

- Frequency varies with temperature

- Frequency varies with supply voltage

- Process variation affects delay

- Higher phase noise than crystal oscillators

- Sensitive to transistor mismatch